#include <OneWire.h>

#include <IRremote.h>

#include <LiquidCrystal.h>

LiquidCrystal lcd(13,12,11,10,9,8);

const int buttonPin = 2;

int buttonState;                    // The key state

IRsend irsend;

OneWire  ds(7);  // on pin 10 (a 4.7K resistor is necessary)

void setup(void)

{

pinMode(buttonPin, INPUT_PULLUP);

Serial.begin(9600);

lcd.begin(16, 2);

lcd.print("Temperature:");

}

void loop(void)

{

byte i;

byte present = 0;

byte type_s;

byte data[12];

byte addr[8];

int celsius, fahrenheit;

if ( !ds.search(addr))

{

Serial.println("No more addresses.");

Serial.println();

ds.reset_search();

delay(250);

return;

}

Serial.print("ROM =");

for( i = 0; i < 8; i++)

{

Serial.write(' ');

Serial.print(addr[i], HEX);

}

if (OneWire::crc8(addr, 7) != addr[7])

{

Serial.println("CRC is not valid!");

return;

}

Serial.println();

// the first ROM byte indicates which chip

switch (addr[0])

{

case 0x10:

Serial.println("  Chip = DS18S20");  // or old DS1820

type_s = 1;

break;

case 0x28:

Serial.println("  Chip = DS18B20");

type_s = 0;

break;

case 0x22:

Serial.println("  Chip = DS1822");

type_s = 0;

break;

default:

Serial.println("Device is not a DS18x20 family device.");

return;

}

ds.reset();

ds.select(addr);

ds.write(0x44, 1);        // start conversion, with parasite power on at the end

delay(1000);     // maybe 750ms is enough, maybe not

// we might do a ds.depower() here, but the reset will take care of it.

present = ds.reset();

ds.select(addr);

ds.write(0xBE);         // Read Scratchpad

Serial.print("  Data = ");

Serial.print(present, HEX);

Serial.print(" ");

for ( i = 0; i < 9; i++)

{           // we need 9 bytes

data[i] = ds.read();

Serial.print(data[i], HEX);

Serial.print(" ");

}

Serial.print(" CRC=");

Serial.print(OneWire::crc8(data, 8), HEX);

Serial.println();

// Convert the data to actual temperature

// because the result is a 16 bit signed integer, it should

// be stored to an "int16_t" type, which is always 16 bits

// even when compiled on a 32 bit processor.

int16_t raw = (data[1] << 8) | data[0];

if (type_s)

{

raw = raw << 3; // 9 bit resolution default

if (data[7] == 0x10)

{

// "count remain" gives full 12 bit resolution

raw = (raw & 0xFFF0) + 12 - data[6];

}

}

else

{

byte cfg = (data[4] & 0x60);

// at lower res, the low bits are undefined, so let's zero them

if (cfg == 0x00) raw = raw & ~7;  // 9 bit resolution, 93.75 ms

else if (cfg == 0x20) raw = raw & ~3; // 10 bit res, 187.5 ms

else if (cfg == 0x40) raw = raw & ~1; // 11 bit res, 375 ms

//// default is 12 bit resolution, 750 ms conversion time

}

Serial.print("  raw:");

Serial.println(raw, HEX);

// Read the key state

buttonState=digitalRead(buttonPin);

// Check button is pressed

if (buttonState==LOW)

{

delay(5);

buttonState=digitalRead(buttonPin);

if(buttonState==LOW)

{

irsend.sendNEC(raw,32);

delay(250);

}

}

celsius =(float)raw / 16;

lcd.setCursor(6, 1);

lcd.print("celsius");

lcd.setCursor(3, 1);

lcd.print(celsius);

fahrenheit =celsius * 1.8 + 32.0;

Serial.print("  Temperature = ");

Serial.print(celsius);

Serial.print(" Celsius, ");

Serial.print(fahrenheit);

Serial.println(" Fahrenheit");

}