NE555 adjustable pulse generator

NE555 adjustable pulse generator is based on NE555 chip, built into an oscillator circuit.

Ⅰ. That principle

The multivibrator composed of 555 timer, and waveform as shown in figure
After turning on the power supply, power supply VDD through R1 and R2 for capacitance C charging, when Uc < 1/3 VDD, oscillator output Vo = 1, discharge tube.After the Uc charge to 2/3 or more VDD, oscillator output Vo flip to 0, the discharge tube conduction, make the discharge end (DIS) grounding, capacitance C through R2 to discharge, bring down the Uc.When reduced to a third or less VDD Uc, oscillator output Vo and flip into 1, discharge tube and as of right now, make the discharge end (DIS) not grounding, power VDD through R1 and R2 and recharging the capacitance C, made up from a third VDD Uc two-thirds VDD, trigger and flip, so the cycle, so as to get continuous variation on the output side Vo oscillation pulse waveform.Pulse width 0.7 R2C TL material, determined by the capacitance C discharge time;TH = 0.7 (R1 + R2) C, is determined by capacitance C charging time, pulse period T material TH + TL

Ⅱ. Directions for use

- The input voltage: DC5 ~ 12 v.The amplitude in: 4.2 V to 11.4 V

- The function and operation:

INPUT DC5~15V

GND

The square wave output port

With short circuit cap link, can output the output 10 k ~ 200 KHZ

With short circuit cap link, can output the output 1 k ~ 10KHZ

With short circuit cap link, can output the output 50~ 1KHZ

With short circuit cap link, can output the output 1 ~ 50HZ

Can adjust output frequency

Can adjust the duty ratio

Output duty cycle can be fine-tuning, duty ratio and frequency is not adjustable, adjustable duty cycle will change the frequency

Ⅲ. Schematic diagram

Ⅳ. List of components

| The serial number | component | label | footprint | value | number |
| --- | --- | --- | --- | --- | --- |
| 1 | resistance | R1 | AXIAL-0.4 | 1.5K | 1 |
| 2 | Ceramics capacitors | C1，C2 | RAD-0.1 | 104 | 2 |
| 3 | Ceramics capacitors | C3 | RAD-0.1 | 102 | 1 |
| 4 | Glowing red LED 5 mm | D1 | DIP |  | 1 |
| 5 | NE555 | U1 | DIP-8 |  | 1 |
| 6 | 8 P IC seat |  |  |  | 1 |
| 7 | Potentiometer | VR1，VR2 |  |  | 2 |
| 8 | 50V 100UF Electrolytic capacitor | C4，C5 | RB.15/.3 | 100UF | 2 |
| 9 | 50V 1UF Electrolytic capacitor | C6 | RB.1/.2 | 1UF | 1 |
| 10 | Row needles |  |  |  | 0.5 |
| 11 | Jumper cap |  |  |  | 1 |
| 12 | PCB |  |  |  | 1 |